# How to install and use the counting foci XTension

Authors : @LPC-UMR6533-CNRS @IMARIS @henri.Payno @yann.Perrot @lydia.Maigne @emmanuel.Delage

# History

17/09/14 : Beta version allowing users to make foci surfaces in correlation with cell (or nucleus) surfaces in order to generate new statistics.

These news statistics include:

- mean number of nuclear foci per cell

- mean area of nuclear foci (um2) per cell

- mean area of nuclear foci (percentage) per cell = sum(fociAreas) / cellArea

- mean number of nuclear foci

# How to install.

This script is based on Imaris and Python. First you should have an operative Imaris. Imaris version should be at least 7.7.1

## Install python

Install the last 2.x.x python which can be loaded here: https://www.python.org/downloads/

If your Imaris version is a 64 bits pick the python 64 bits.

Python 3.x.x is not working for this script.

## Install third libraries

In order to successfully execute this script you will need the following libraries:

- Scipy

- Numpy

- Six : https://pypi.python.org/pypi/six. For this library you don't have any installer, you just have to download it and untar/unzip files to a dedicated folder.

- Dateutil : https://pypi.python.org/pypi/python-dateutil. For this library you don't have any installer, you just have to download it and untar/unzip files to a dedicated folder.

- Pyparsing : http://pyparsing.wikispaces.com/Download+and+Installation

- Matplotlib

- Pick the compatible version with your python version (2.x, 32 or 64 bits). You can download it here: http://matplotlib.org/downloads.html

- Pylab : is already included in Matplolib.

Make sure your libraries insure compatibility with Imaris ( 32, 64 bits ) and your python version ( 2.x )

Many libraries compiled for 64 bits can be fond here : http://www.lfd.uci.edu/~gohlke/pythonlibs

## Update your “PATH” environmental variable.

Under windows 8 you can access to the environmental variable through ‘configuration pannel/System /Advanced System Settings/Environmental variables’. Then edit “Path”. Here is a small tutorial:

http://www.computerhope.com/issues/ch000549.htm

“PATH” environmental variable have to be updated, including the path to the Python third libraries ( Dateutil... )

## Install XTCountingScript

Set the python script “XTCountingFoci.py” to the IMARIS python XTension folder “C:\IMARIS\XT\python” if you kept the default settings.

## Link Imaris and Python

If it is your first time using Python script with Imaris you should set your python environment in Edit/Preference/CustomTools.

- Set python script folder (1) in XTensionFolders

- Set python executable path in Python Application : “C:/Python27/python.exe” is kept as default.

# How to use

## Load data

First load your data to Imaris. In our example we have two channels, one (green) for focis and one for nuclei (blue).

Figure 1 : Example of loaded data

## Create Surfaces

This script is based on two surfaces, one for nuclei or cell and one for focis. You should create these two surfaces using Imaris “Add new surfaces” tools.

Here is an example to create focis surfaces:

Figure 2 : Launching "Add new surfaces" for focis

Figure 3 : During creation of focis surfaces

Figure 4 : Created foci surfaces

Then we do the same for nuclei:

Figure 5 : Nuclei surfaces creation

After the creation of this two sets of surfaces you should obtain a result near this one:

Figure 6 : Result of surfaces creation

## Using XTcountingFoci script

Once both surfaces sets are created you can launch the Imaris XTCountingFoci script accesible throught: “ImageProcessing/ImarisOpen/Counting foci”

Figure 7 : Launching script

Then you just have to follow instructions :

- First select the foci surfaces and press enter.

Figure 8 : Selection of foci surfaces

- Second: select nuclei or cell surfaces created previously.

Figure 9 : Selection of nuclei surfaces

Then press enter. The script should launch. After processing result appears:

Figure 10 : Results

Results contain the unassociated foci that can appear if none are stored on defined nuclei/cells.

It also includes the mean foci area on cell and the surface occupyied  by foci on cells (mean relative size). You can find other information such as the mean number of foci per cells/nuclei and the mean sum intensity of focis.

Those results are also stored on the Imaris “Statistics” tab of nuclei/cell surfaces.

Figure 11 : Results on Nuclei surfaces "statistics" tab

The association made can be displayed by setting the script value “drawAssociation” to True.

Figure 12 : Association made

# Limitation

For now nuclei/cell delimitation are based on a simple bounding box, on the future we would like to use a more realistic boundary such as convex-hull.